JP BARON DATEC SYSTEM PROBLEM TABLE COMPAK SPORTING

The table is to be used as a guide to resolving issues experienced by the Referee when operating the DATEC Line System equipment for FITASC Compak Sporting.

| Equipment | Problem | Referee or Assistants Action |
| --- | --- | --- |
| Visual Display Unit | The VDU goes blank | Switch the VDU off, wait 10 seconds switch the VDU back on the VDU should return to the results. |
| Main Control Unit | The Main Control Unit screen is blank | Put down back button of datec controller “wake up”. |
| Hand Release Unit | The Hand Release screen is blank | Press once the M/A switch the unit will “wake up”. |
| Hand Release Unit | The release unit does not display the correct Compak channel (Range) | Press the +(9) or –(0) to increase the number or decrease the number to indicate the correct range |
| Main Release Unit | The microphones’ are too sensitive or not sensitive enough to release the target(s) | Press the volume +/- to increase or decrease the sensitivity. (You see % on screen, down right) 100 = maximum sensitivity. |
| Referee Hand Controller | The microphone lights are not working | On the referee hand controller select the Pause/On switch to ON. |
| Main Control Unit | The next in line competitor is not there (No Show) | On the Main Controller press “Menu” one time, display will show either next shooter or Advance, press “Validate” the system will move to the next competitor on the time sheet. Note: this must only be done when the system is ready for position one as displayed on the VDU by a flashing symbol at position one (1) score line. |
| Referee Hand Controller | The target is considered a ‘No Target’ with NO shot fired | Do NOT press the “NO Target” on the hand controller. The system will automatically reset. |
| Main Control Unit | The referee accidentally pressed Zero when the target was killed | On the main control unit press (1) No Bird, (2) Advance. |
| Main Control Unit | The referee accidentally pressed Zero twice (2) when it should have been once | On the Main Controller press: (1) No Bird, (2) Advance (3) Zero. |
| Main Control Unit | When the last shooter moves from position 1 to position 2 (then 3, 4 and 5) | On the Main Controller press menu 4 times this will scroll on the display to “end sequence” or words to that effect then press “Validate” this will move the system on to position 2. Note: this must be done at every position as the last competitor moves to their next position |
| Main Control Unit | The referee accidentally press NO Bird | On the Main Controller press: (1) Advance |
| Main Control Unit | The competitor fails to fire at a single target | On the Main Controller press: (1) Advance, (2) Zero. |
| Main Control Unit | The competitor fails to fire at the second target of a double (first shot killed) | On the Main Controller press: (1) Advance, (2) Zero. |
| Main Control Unit | The competitor fails to fire at the second target of a double (first shot missed) | On the Main Controller press: (1) Advance, (2) Zero, (3) Zero. |
| Main Control Unit | The computer advances past the next position to shoot i.e. moves from 2 to 4 (miss shooter 3) | On the Main Control Unit press (1) “No Bird” then correct button of shooter 3 to enable light and (2) Advance to enter on post 3. |

Notes:

- The Zero button on the Referees small hand controller should be given a positive push then release when scoring zero.
You must put a 2nd zero only when you listen the first zero.

- Do NOT press the no bird button if a shot has not been fired the system will automatically No Bird and reset back to that position.

- When showing targets switch the Pause switch to Off to disable the microphone(s)